H Claus 11/18/25 V1

## Suggested serial commands for OSLUV controller

### Set/ Get

“S:” set

- Set value, return entire command +“:OK”

“G:” get

- Return entire command +”:value”

In case of error: return “Err” instead of “OK” or value

### Values

Structure: 1 character (+ “:” + value)

#### Values to be set/get

- Lamp on/ off

  - “L” +”:” + “1” (on)

  - “L” +”:” + “0”  (off)

- Dim level

  - “D” +”:” + “20” (for dim level 20% - up to 3 digits)

    - Accepted dim values: 20; 40; 70; 100

Example:

set dim level to 40%:  S:D:40

return: S:D:40:OK

get dim level: G:D

return: G:D:40

turn lamp on: S:L:1

return (after checking status check) S:L:1:OK